# FOR THE INSTRUCTOR

PURPOSE:	To teach the user how to use the com6srvr.exe file with Visual Basic;

NOT to teach them how to program Visual Basic itself.

TIME:		2 hours.

TIMING:	After they have learned how to use the 6K’s Ethernet capability.

PROBLEMS:	User doesn’t read and follow the written instructions (especially the Notes).

Not consistent in the naming or spelling of objects and tools.

Commport is the wrong number.

Ethernet is the wrong address.

User used the wrong tool from the toolbox.

Something is wrong with the pointer in the DLL, but I don’t know what!

# USING COMPUMOTOR’S 6K WITH VISUAL BASIC

After opening Visual Basic 6.0, the screen should consist of five main parts:

- Toolbar—across the top.

- Toolbox—on the left.

- Form Designer workspace—large window in the middle.

- Project Explorer—on the top-right.  (NOTICE: “Name (filename.frm)”)

- Properties Window—on the bottom-right.

- Add a Module.  Right-click in the Project Explorer (top-right window) and Add a Module.

- Create a Commserver Object within the Module.  Double-click on the Module in the Project Explorer.  In the window that pops up, type in:

Global C6K As Object

## Global ConnectReturnValue As Integer  (NOTICE: No spaces in the variable.)

- Name the Module.  In the Properties Window (bottom-right window), name the Module “modGlobal”.

- Name the Form.  Click on the Form window.  In the Properties Window, next to Name, enter “frmMain”.

- Save the Project.  Go to the File menu and select SAVE PROJECT.

When asked to name the Module, enter “Global” as the name.

When asked about the Form, enter “Communication”.

When asked about the Project, enter “Basic Communication”.

(Note that the names in the Project Explorer window change.)

- Create a Connect Button.  Click on the Command Button in the Toolbox (left window).  Then, click-and-drag on the Form to create the Button.  In the Properties Window, make the Name “cmdConnect” and the Caption “CONNECT”.

- Create Connection Code.  Double-click on the Connect Command Button.  The Code Window will open with “Private Sub cmdConnect_Click ( )” and “End Sub” already entered.  Between them, type in:

### Dim CommPort As Integer

Set C6K = CreateObject (“COM6SRVR.RS232”)

CommPort = 1

ConnectReturnValue = C6K.Connect (CommPort)

(NOTICE: Make sure the 6K is connected to Port 1 on the computer.  Or, if using Comm 2 on the computer, make CommPort = 2.)

- Return to the Form Window.  In the Project Explorer’s toolbar, click on the middle icon: View Object.

- Create Disconnect Button.  Add a new Command Button to the form.  Name it “cmdDisconnect” and make its Caption “DISCONNECT”.

- Create Disconnect Code.  Double-click on the Disconnect Command Button and add a line of code:

Set C6K = Nothing

- Create Send Text Box.  In the Toolbox, click on the Text Box.  Click-and-drag a Text Box on the Form.  Call it “txtSend” and put “TREV” in the Text Property.

- Create Send Button.  Add a new Command Button to the form.  Name it “cmdSend” and Caption it with “SEND”.

- Create Send Code.  Double-click on the Send Button and add some code:

Dim Temp As String

Temp = txtSend.Text & ":"  (NOTE:  Spaces before and after the & sign.)

C6K.write (Temp)

- Create Receive Text Box.  Add a new Text Box to the Form.  Call it “txtReceive” and remove any text from the Text Property.

- Create Receive Button.  Add a new Command Button called “cmdReceive” with a Caption of “RECEIVE”.

- Create Receive Code.  Double-click on the Receive Button and add a line of code:

txtReceive.Text = C6K.read

- Create Clear Button.  Add a new Command Button called “cmdClear” with a Caption of “CLEAR”

- Create Clear Code.  Double-click on the Clear Button and two lines of code:

txtSend.Text = ""

txtReceive.Text = ""

- Run the Program.  On the Toolbar, press the Start icon that looks like a Play button.  Connect the program to the 6K with the Connect Button.  Then, send a TREV to the 6K by pushing the Send Button.  Now, get the response by pushing the Receive Button.  (NOTICE: Connect must be pushed first prior to any other commands.)

- Stop the Program.  Either close the window with the “X” in the top-right corner, or hit the Stop button on the Toolbar.

- Create Time Delay Function.  Add a new Module.  Call the Module “modDelay”.  In the Module, add some lines of code:

Declare Function timeGetTime Lib "winmm.dll" () As Long

‘Visual Basic will automatically find this DLL on your computer.

‘Units are in milli-seconds.

Sub Delay(ByVal dTime As Long)

'Dim sTime As Long

sTime& = timeGetTime()

Do

dummy% = DoEvents

Loop While ((timeGetTime() - sTime&) <= dTime)

End Sub

- Create Send/Receive Button.  Add a new Command Button called “cmdSendRec” with a Caption of “SEND / RECEIVE”.

- Create Send/Receive Code.  Double-click on the Send/Receive Button and a few lines of code:

C6K.write (txtSend.Text & ":")   (NOTE: Spaces before and after the & sign.)

Delay (Val(txtDelay.Text))

txtReceive.Text = C6K.read

- Create Time Delay Text.  Add a Label with a Caption of “Time Delay Between Send and Receive:”  Add a new Text Box, place it next to the Label, and call it “txtDelay” with Text of “50”.  Add a new Label with a Caption of “ms” and put it after the Text Box.

- Run the Program.  Try out the Send/Receive Button.  Change the 50 to a different number to find out what works and what does not.  (NOTICE: Connect must be used to initialize the communication first.)

- Create an RS232 Option Button.  Add an Option Button from the Tool Box.  Name it “optRS232” with a Caption of “RS232”.

- Create an Ethernet Option Button.  Add a new Option Button from the Tool Box.  Name it “optEthernet” with a Caption of “ETHERNET”.

- Create a Commport Text Box.  Add a new Text Box.  Call it “txtCommport” with Text of “1”.

- Create an Ethernet Address Text Box.  Add a new Text Box.  Call it “txtAddress” with Text of “192.168.10.30”.  (This is the 6K’s address, not the computer’s.) Change this text to an address that is appropriate to be used with the computer that is being used.  Find the computer’s IP address by going to Start => Settings => Control Panel => Network => TCP/IP => Properties.  Add either one or one hundred to the last number of this Address.

- Create a Go Button.  Add a new Command Button.  Call it “cmdGo” with a Caption of “GO”.

- Create Go Code.  Double-click on the Go Button and add a line of code:

C6K.write ("GO1:")

- Modify the Connection Code.  Double-click on the Connect Button and change the code to include all of these lines:

Dim Address As String

Dim CommPort As Integer

If optEthernet.Value Then

Set C6K = CreateObject("com6srvr.net")

Address = txtAddress.Text

ConnectReturnValue = C6K.Connect(Address)

Else

Set C6K = CreateObject("COM6SRVR.RS232")

CommPort = Val(txtCommport.Text)

ConnectReturnValue = C6K.Connect(CommPort)

End If

- Run the Program.  Before trying the Ethernet connection, send some setup commands over the RS232.  Enter  “NTADDR192,168,10,30: NTMASK255,255,255,0” then push the Send button.  Now enter “NTFEN1” in the Send box and send it.  Now change over to the Ethernet and connect with it.